CSPC · ICTU

SERVER ROOM SENSOR

What the light means

When you switch it on

|  | Blue — Starting up. Wait about 20 seconds. |
| --- | --- |
|  | Purple — Press BOOT now to change the WiFi. 4 seconds only. |
|  | Red — Wrong WiFi password, or no network found. |
|  | Orange — On WiFi, but it cannot reach the server. |
|  | Green — Ready. Nothing to do. |

While it is working

|  | Green — Everything is normal. |
| --- | --- |
|  | Green with a blue blink — Not reaching the dashboard. Nothing is lost. |
|  | Bright blue — The room is too cold. |
|  | Yellow — Getting hot, humid or smoky. |
|  | Orange — Humidity is too high. |
|  | Red — DANGER — heat or smoke. Check the room. |

What the sound means

- One long continuous tone — SMOKE. Act now.

- Fast beeping — too hot, or too humid.

- Slow beeping — warning.

What is plugged in

| CH | Smoke sensor | Aircon remote |
| --- | --- | --- |
| 1 | Fitted | Fitted |
| 2 | Fitted | Fitted |
| 3 | Empty | Empty |
| 4 | Empty | Empty |

CH3 and CH4 are spare. A technician can fit them — ask ICTU. The dashboard uses these same CH numbers.

To change the WiFi

- Plug it in. Do not press anything yet.

- Wait about 4 seconds, until the light turns purple.

- While it is purple, press the small BOOT button once.

- On your phone, join the WiFi CSPC-ICTU-Sensor (no password). A setup page opens by itself — pick a 2.4 GHz network.

Never hold the BOOT button while plugging it in. It will stop working. Unplug and start again.

Never

- Open it or touch the wiring while it is plugged in.

- Take out the small round battery inside.

- Connect it to a 5 GHz WiFi network.

Do not open the box. Anything not on this sticker — contact ICTU or the development team.