# What IR Data to Learn from the AC Remote

A capture guide for the aircon IR control feature.

# STATUS — updated after first captures

Protocol = UNKNOWN (confirmed). The IRremoteESP8266 library does NOT recognize this remote, so there is no library shortcut. We reverse-engineer the 64-bit packet ourselves, then synthesize frames in firmware (sent via sendRaw, which already works on your AC).

==> Skip the "known protocol" branch. Go straight to the Capture List below. The Cool temp sweep is the single most important batch — it cracks the temperature field AND the checksum at once.

Re-capture needed: the earlier FAN_LVL2 frame was corrupt (its footer decoded to 80 70 instead of the constant E0 10 — a bit was dropped). Capture COOL 24 HIGH again.

# What your first 10 captures already told us

The packet is 64 bits with this structure (LSB-first bytes):

byte:  0    1      2    3    4    5       6    7
       14   <btn>  ..   ..   ..   <cksum> E0   10
       ^^header                  ^^checksum  ^^footer (constant)

byte 0 = 0x14 and bytes 6-7 = E0 10 are constant on every frame (header / footer).

byte 5 is the checksum (changes every frame) — NOT a simple byte sum/XOR; needs clean incremental samples to crack.

byte 1 looks like a "which button was pressed" code, not AC state.

bytes 3-4 carry temp/mode/fan, but were confounded in the first captures (several settings changed at once, at unknown temps).

# Capture rules (so the data is decodable)

Change only ONE setting between captures. Keep everything else fixed.

Label every capture with the full display: MODE TEMP FAN.

Don't touch swing / sleep / timer — leave them fixed for all captures.

Capture each state twice. If the two frames differ, the remote has a toggle/repeat bit.

Frequency stays 38 kHz (the IRLearner.ino default) unless the AC doesn't respond.

# The Capture List (~18 frames)

| # | Group | Capture | Keep fixed |
| --- | --- | --- | --- |
| 1 | Cool temp sweep [PRIORITY] | COOL <t> LOW for every temp the remote shows: 18, 19, 20 ... up to max, one per degree | Mode = Cool, Fan = Low |
| 2 | Modes | DRY 24 LOW, FAN 24 LOW | Temp = 24, Fan = Low |
| 3 | Fan speeds | COOL 24 HIGH (+ COOL 24 AUTO if it has Auto fan) | Mode = Cool, Temp = 24 |
| 4 | Turbo | COOL 24 TURBO | Mode = Cool, Temp = 24 |
| 5 | Re-capture | FAN_LVL2 = COOL 24 HIGH (previous one was corrupt) | Mode = Cool, Temp = 24 |
| 6 | Power | OFF, ON | (already captured, OK) |

The Cool temp sweep is the priority. Capturing the whole range also reveals the true lowest temperature the remote actually reaches (confirms 22 vs 18). Only capture values the display will actually show.

For Dry and Fan modes, also note what temperature the display shows (many ACs lock or hide the temp in those modes).

# How to label / paste the captures

Title each block with its exact state:

COOL 18 LOW:
uint16_t rawData[] = { 9000, 4500, ... };

COOL 19 LOW:
uint16_t rawData[] = { 9000, 4500, ... };

DRY 24 LOW:
uint16_t rawData[] = { 9000, 4500, ... };

# What each group buys us

Temp sweep -> locates the temperature bits + cracks the checksum -> firmware can synthesize ANY temperature.

Modes + Fan + Turbo -> identifies those bit fields.

From ~18 captures I can generate every Cool/Dry/Fan x temp x fan combination instead of capturing all ~70 individually.

Fallback: if the checksum proves too hard, the same temp-sweep frames double as a raw lookup table (proven sendRaw path). So the sweep is the right next step either way.

# Next action

Capture the Cool temp sweep (18 -> max, one frame per degree, each labeled), plus the mode/fan/turbo frames and the FAN_LVL2 re-capture. Paste them labeled as shown above.