# Dev MikroTik Setup — for building Network Monitoring

Goal: get the small dev MikroTik ready so the backend can connect to it over the RouterOS API (read-only) and we can build + live-test the feature end to end — without waiting on the campus questionnaire. See mikrotik-monitoring.md for the design.

What the monitoring backend needs from the router, and what each step here provides:

| The app needs | Provided by |
| --- | --- |
| A reachable IP for the router | Steps 1–2 |
| RouterOS version (v6/v7) | Step 2 |
| The API service enabled (port 8728) | Step 3 |
| A read-only login (username + password) | Step 4 |
| The port reachable from the backend host | Steps 5, 7 |
| The interface names (your test "buildings") | Step 6 |
| Values for the DB seed + .env | Step 8 |

⚠️ This is a DEV box on a trusted LAN, so we keep it simple (plain API on 8728, no TLS). Production hardening (API-SSL/8729, firewalled to the backend only) is a go-live step, not this.

# What you need

The dev MikroTik + its power adapter, and an Ethernet cable.

A PC on the same network as the router (this will also run the backend).

WinBox (easiest) — download from https://mikrotik.com/download (Windows). You can also use WebFig (browser) or SSH / the terminal — every step shows the CLI command, which works the same in WinBox's New Terminal, WebFig's Terminal, or SSH.

# Step 1 — Power on and connect

Plug the router in. Connect an Ethernet cable from your PC to one of the router's LAN ports (usually NOT ether1 — that's typically the WAN/uplink port).

A factory-default MikroTik hands out DHCP on 192.168.88.0/24 and is reachable at 192.168.88.1. Your PC should get a 192.168.88.x address automatically.

Open WinBox → the Neighbors tab → it discovers the router (you can connect by MAC even if the IP is unknown). Default login is user 'admin' with no password (newer units may have a password on a sticker / ask you to set one on first login).

If it's not factory default and you don't know the IP: WinBox Neighbors (connect by MAC), or reset to defaults (hold the RES button while powering on until the LED flashes) — only if it's a throwaway dev box.

# Step 2 — Check the RouterOS version + identity

/system resource print ; shows version (v6 vs v7), board-name, cpu, memory, uptime /system identity print ; the router's name /ip address print ; its IP address(es)

Note the version — 7.x means we can also use the dependency-free REST API; node-routeros works for both, so either way you're fine.

Give it a fixed identity: /system identity set name=dev-mikrotik

# Step 3 — Enable the RouterOS API

The classic API listens on TCP 8728 (8729 = API-SSL, needs a certificate — skip for dev).

CLI:

/ip service print ; see which services are enabled /ip service enable api ; turn on the API on 8728 /ip service print ; confirm 'api' is no longer disabled (X)

WinBox/WebFig: IP → Services → select 'api' → click Enable. Leave port 8728.

# Step 4 — Create a read-only monitoring user

Never let monitoring use 'admin'. Make a dedicated group with read + api only (no write), then a user in it. 'test' is for the ping tool (latency add-on later); 'rest-api' lets you try REST.

/user group add name=monitoring policy=read,api,rest-api,test,winbox \ comment="read-only for CSPC monitoring" /user add name=monitor-ro group=monitoring password="ChooseAStrongPasswordHere" \ comment="CSPC monitoring (read-only)"

Verify:

/user print ; monitor-ro should be in group 'monitoring'

Why these policies: read + api is the minimum the poller needs. write/policy are NOT granted. With this account a leaked password can only read status.

# Step 5 — (Recommended) restrict the API to the backend host

Lock the API so only your backend PC can talk to it:

/ip service set api address=192.168.88.0/24 ; or a single host, e.g. 192.168.88.50/32

Replace with your backend host's IP/subnet. On a dev LAN the subnet is fine.

# Step 6 — List the interfaces (your test "buildings")

Each building = one port. For dev, the physical ports stand in for buildings.

/interface print ; the names: ether1, ether2, sfp1, ... /interface ethernet print ; link speed / state per port

Write down the names — these go in network_interfaces as interface_name, each labeled with a fake building (e.g. ether2 → "Test Building A"). Empty ports show link down / zero traffic, which is fine; plug a device into one to see real throughput.

# Step 7 — Verify the backend host can reach the API

From the backend PC (PowerShell):

Test-NetConnection 192.168.88.1 -Port 8728 ; TcpTestSucceeded : True = good

Use the router's real IP. If it fails: check Step 3 (api enabled), Step 5 (address restriction), and any firewall on the router (/ip firewall filter print) or the PC.

# Step 8 — Record the values for the app

Plug these into the dev seed in migrations/2026-06-20_mikrotik_device.sql (uncomment the template) and into backend/.env:

DB seed (dev):

| Field | Dev value |
| --- | --- |
| devices.ip_address | the router IP (e.g. 192.168.88.1) |
| devices.device_name | dev-mikrotik |
| devices.device_type | mikrotik |
| devices.location | Dev Bench |
| mikrotik_devices.api_port | 8728 |
| mikrotik_devices.use_tls | 0 |
| mikrotik_devices.api_username | monitor-ro |
| network_interfaces rows | ether2→"Test Building A", ether3→"Test Building B", … |

.env: add the encryption key the app uses to store the API password (32-byte hex):

MIKROTIK_ENC_KEY=<run: node -e "console.log(require('crypto').randomBytes(32).toString('hex'))"> MIKROTIK_POLL_INTERVAL_MS=30000

The password isn't seeded as plaintext — you'll enter it once via the dashboard admin form (Phase 3), which encrypts it with MIKROTIK_ENC_KEY before storing in api_password. For an early Phase-2 bring-up before that form exists, we'll add a tiny one-off encrypt helper.

# Optional — simulate multiple buildings on one router

For dev you usually don't need this. To test per-building client counts, give two ports their own subnet + DHCP server so leases group per "building":

/ip address add address=10.10.2.1/24 interface=ether2 /ip pool add name=bldgA ranges=10.10.2.10-10.10.2.100 /ip dhcp-server add name=dhcpA interface=ether2 address-pool=bldgA disabled=no /ip dhcp-server network add address=10.10.2.0/24 gateway=10.10.2.1

Repeat on ether3 with 10.10.3.0/24 for a second building. Plug a laptop/phone into each to get DHCP leases the poller can count per building.

# Security notes (dev vs production)

Dev: plain API (8728) on a trusted LAN, address-restricted to the backend, read-only user. Fine.

Production (campus MikroTik, later): prefer API-SSL (8729) with a certificate, firewall the API to the backend host only, and again a read-only user. Captured in the questionnaire.

# Quick troubleshooting

| Symptom | Check |
| --- | --- |
| Can't find the router | WinBox Neighbors (connect by MAC); confirm cable in a LAN port |
| Test-NetConnection … 8728 fails | /ip service print (api enabled?), address restriction (Step 5), router/PC firewall |
| Login refused for monitor-ro | /user print (right group?), retype password; group has api policy |
| Want the REST API instead (v7) | enable www-ssl + use https://<ip>/rest; rest-api policy granted in Step 4 |