# Data Flow Diagrams

Server Infrastructure Monitoring System · CSPC — ICTU · Gane–Sarson notation

| File | Figure | Content |
| --- | --- | --- |
| dfd-level0-clean.drawio | Figure 3 | Context diagram — the whole system as ONE process, 9 external entities, 16 composite flows |
| dfd-level1-clean.drawio | Figure 4 | The same system decomposed into 6 processes, 10 data stores, 29 boundary flows |

# Manuscript narrative

## Figure 3 — Level 0 (context)

Figure 3 presents the Level 0, or context, data flow diagram of the developed system, treating the system as a single process bounded by nine external entities. Two are human actors: Admin and IT Staff exchange sign-in, approvals, alert-rule configuration, gas-sensor setup, acknowledgements, and air-conditioner control requests for a session, role, and live dashboards, alerts, reports, and forecasts in return. Four are the monitored sources — the Server Room, Monitored Server, Router/MikroTik, and UPS Unit — each supplying its respective reading (temperature/humidity/gas, CPU/memory/disk, traffic and link state, or battery/load/runtime) in exchange for a query or an agent token. The Server Room additionally reports the pin map of its MQ-2 gas channels and receives in return the gas-sensor configuration that determines which of those channels are read and how each is labelled. The remaining two entities support authentication and notification: Google Workspace exchanges an authorisation code for a verified identity, and the Email Recipient receives alert and report e-mail. Figure 4 decomposes this single process into its constituent functions.

The Server Room sentence carries two flows that did not exist before the MQ-2 sensors became managed data (migrations/2026-09-17_gas_sensors.sql): the room now reports which ADC pin each gas channel sits on, and receives which channels are wired and what each is called. It is the only monitored source whose return flow is a configuration rather than a poll, which is why it gets a sentence of its own rather than being folded into the list.

## Figure 4 — Level 1

Figure 4 decomposes the context process of Figure 3 into six processes and ten data stores. Process 1.0, Authenticate and Authorize, exchanges the Google authorization code and sign-in requests for a verified identity, session, and role, and records each account in D1 users and each enrollment key in D9 agent_install_keys. Process 2.0, Collect Monitoring Data, receives readings from all four monitored sources, writes each sample to D3 measurements and D7 backup files, and updates device status in D2 devices. It also holds the MQ-2 gas sensors as managed data: Admin supplies each channel’s label and whether it is enabled, which Process 2.0 stores in D10 gas_sensors and reads back on every reading to decide which channels count and how each is named; the wiring is pushed to the Server Room as a gas channel configuration, the Server Room reports the pin map of those channels in return, and the change is logged to D6 system_logs. Process 3.0, Evaluate Rules and Notify, checks each incoming sample against the thresholds held in D8 alert_rules, writes new alerts to D4 alerts, and returns alert notifications to IT Staff and alert e-mail to the Email Recipient; it also forwards a temperature-zone change to Process 4.0. Process 4.0, Control Air Conditioning, combines that zone change with manual on/off commands from IT Staff to issue an IR command to the Air Conditioner Unit and log the resulting unit state to D2 and D6 system_logs. Process 5.0, Generate Reports, draws period measurements and alerts from D3 and D4 to produce the CSV/PDF file recorded in D5 reports and e-mailed to the Email Recipient. Process 6.0, Analyse and Forecast, queries a history window from D3 to return forecasts, anomalies, and recommendations to IT Staff, and feeds a forecast alert back into Process 3.0. All rule changes, acknowledgements, and report actions are additionally logged to D6 system_logs.

The added Process 2.0 sentence carries all six gas-sensor flows at this level — Admin → 2.0, 2.0 → D10, D10 → 2.0, 2.0 → Server Room, Server Room → 2.0, and 2.0 → D6. The store count moved from nine to ten because D10 gas_sensors is new; every other word is as originally written.

# Reading the diagrams

Processes. 1.0 Authenticate & Authorise · 2.0 Collect Monitoring Data · 3.0 Evaluate Rules & Notify · 4.0 Control Air Conditioning · 5.0 Generate Reports · 6.0 Analyse & Forecast.

Data stores. D1 users · D2 devices · D3 measurements (InfluxDB) · D4 alerts · D5 reports · D6 system_logs · D7 backup files · D8 alert_rules · D9 agent_install_keys · D10 gas_sensors.

The asterisk marks a duplicated symbol — the same entity or store drawn more than once so that no flow line has to cross another. There is still only one IT Staff (drawn six times, once per process it talks to), one Admin (three times), one system_logs (five times). The D-number is the identity, not the box: every box labelled D6 is the same table. Count entities by name, never by symbol, or Level 1 appears to have 17 entities against Level 0's 9.

Entities that appear exactly once carry no asterisk: Server Room, Monitored Server, Router/MikroTik, UPS Unit, Air Conditioner Unit, Google Workspace, and stores D1, D5, D7, D8, D9, D10.

Layout convention. One horizontal band per process: external entities on the left, the process in the centre, data stores on the right. Every flow has its own connection point and its own straight line — no two flows share a segment. Three flows run process-to-process (2.0→3.0 evaluated sample, 2.0→4.0 temperature zone change, 6.0→3.0 forecast alert); each is routed in its own reserved corridor.

# Balance

The two levels are balanced: every flow crossing the system boundary at Level 1 rolls up into exactly one Level 0 composite, and every Level 0 flow has at least one Level 1 counterpart.

| Measure | Value |
| --- | --- |
| Level 0 flows | 16 |
| Level 1 boundary flows | 29 (1.0: 8 · 2.0: 11 · 3.0: 4 · 4.0: 2 · 5.0: 3 · 6.0: 1) |
| Level 1 data-store flows | 23 internal — correctly absent from Level 0 |
| Level 1 process-to-process flows | 3 internal — correctly absent from Level 0 |
| External entities | 9 vs 9, identical sets |

Roll-up, by entity and direction:

| Flow | L1 | L0 | Level 1 detail |
| --- | --- | --- | --- |
| Admin → system | 5 | 1 | sign-in · approve/reject registration · issue/revoke install key · threshold rule edits · gas-sensor label & enable |
| system → Admin | 1 | 1 | session & role |
| IT Staff → system | 4 | 1 | sign-in · acknowledge/resolve · manual on/off · report request |
| system → IT Staff | 5 | 1 | session & role · live metrics, sensor readings & labels · alert notification · report file · forecast, anomaly & recommendation |
| Server Room → system | 2 | 1 | temperature, humidity, gas · gas channel pin map |
| system → Server Room | 1 | 1 | gas channel configuration |
| Monitored Server ↔ system | 1 / 1 | 1 / 1 | CPU/memory/disk sample · agent token |
| Router/MikroTik ↔ system | 1 / 1 | 1 / 1 | traffic & link state · SNMP/RouterOS query |
| UPS Unit ↔ system | 1 / 1 | 1 / 1 | battery, load & runtime · SNMP query |
| system → Air Conditioner | 1 | 1 | IR command |
| Google Workspace ↔ system | 1 / 1 | 1 / 1 | verified identity · authorisation code |
| system → Email Recipient | 2 | 1 | alert e-mail · report e-mail + PDF |

Why the counts differ and it is still balanced. Level 0 has one process, so four separate IT Staff inputs would be four arrows into the same box, saying nothing a single arrow does not. They are bundled into one composite arrow whose label lists them — which is why f03 reads “sign-in, acknowledgements, AC control & report requests”. Level 1 splits the process into six, so each item finally has a distinct process to land on. Data stores and process-to-process flows never appear at Level 0 because they do not cross the system boundary.

Maintaining it. A feature that adds a flow crossing the system boundary changes both files. A flow that only touches a data store changes Level 1 alone. The legend note inside each .drawio states its own counts — update them, or the diagrams assert a balance they no longer have.

# Exporting to PDF

File → Export as → PDF, then:

| Setting | Value | Why |
| --- | --- | --- |
| Zoom | 100% | Vector output — zoom changes the nominal page size only, never quality |
| Border Width | 20 | The only margin you get; cropping discards the page margins in the file |
| Size | Diagram | Crops the page to the drawing's bounding box → one page, never tiled |
| Transparent Background | off | Transparent embeds into Word with a grey block behind it |
| Shadow / Grid | off | — |
| Include a copy of my diagram | on | Embeds the .drawio XML so the PDF reopens as an editable diagram |

Do not choose a paper size or “Fit to Page” for Level 1. Its content is 1840 × 2890 px (≈ 19.2 × 30.1 in at 100%); against A4, draw.io tiles it across six sheets with flow lines cut mid-span. Size: Diagram keeps it whole, and scaling it down at placement is lossless.

Printed sizes, for planning:

| Diagram | Content (px) | Aspect | Fits |
| --- | --- | --- | --- |
| Level 0 | 1420 × 949 | 1.50 : 1 landscape | Folio landscape (1.53 : 1) — labels ≈ 5.5 pt |
| Level 1 | 1840 × 2890 | 1 : 1.57 portrait | Folio portrait (1 : 1.53) — labels ≈ 4.5 pt |

Level 1's labels are legible on screen and marginal on paper. If it goes into the printed manuscript, print that page on A3 (labels rise to ≈ 6 pt) or treat it as a fold-out. Splitting it into two half-diagrams would cost the single-page balance argument above.